Faculty of Computing

Computer Architecture

Spring 2025

LAB # 5

Instructor

Ayesha Akram

TF, Faculty of Computing,

Riphah International University, Islamabad

Learning Objectives:

- Understand the basics of CPU Sim software

- Learn how to set up and configure the environment

- Explore basic CPU architecture components

- Develop and test simple assembly programs

- Analyze the Fetch-Decode-Execute cycle through simulations

Essential Tools in Our Lab:

- Computer System: The main machine

- CPU Sim: CPU Sim is a tool used for simulating simple CPU architectures, helping students understand processor design, instruction execution, and debugging.

Introduction to CPU Sim

CPU Sim is an educational software used for simulating the functioning of a CPU at the microarchitecture level. It allows to design and test simple CPU architectures, write assembly language programs, and understand instruction execution cycles.

Key Features of CPU Sim

- Custom CPU Design: Users can define their own CPU components, such as registers, memory, and ALU.

- Instruction Set Simulation: Allows defining and testing assembly language instructions.

- Memory Management: Simulates memory read/write operations and addressing modes.

- Execution Cycle Analysis: Visualizes fetch, decode, and execute cycles for each instruction.

- Step-by-Step Debugging: Helps students analyze instruction execution and troubleshoot errors.

Computer Architecture

- Instruction Cycle Understanding: Demonstrates the role of the control unit, ALU, and registers.

- Pipeline Processing: Helps visualize how pipelining works in instruction execution.

- Memory Hierarchy Simulation: Illustrates how registers, cache, and RAM interact.

- Microprogramming: Allows experimentation with microinstructions for CPU control.

# Download and Install CPU Sim

- Download the JRE compatible with your operating system.

https://centrexmanager.frontier.com/webstart/webstartHelp.jsp

- Visit the official CPU Sim website.

https://www.cs.colby.edu/djskrien/CPUSim/

## Introduction:

CPU SIM is a software tool used to simulate a basic CPU architecture, helping students understand how instructions are fetched and executed.

In this lab, we will:

- Define registers required for execution.

- Implement the Fetch Cycle.

- Click on the batch file and the window of CPU Sim is open.

The screen shows a CPU simulation interface with the following components:

- Top Menu Bar: Options like File, Edit, Execute, and Help.

- Registers Panel (Left): Lists registers (pc, ir, mar, etc.) with their bit width and current values (all 0).

- Code Editor (Center): Displays an assembly program.

- Memory Panel (Right): Shows memory addresses and data.

- Program Explanation (Bottom): Comments describe the program logic and storage of the final sum.

This setup is for writing, testing, and debugging assembly programs.

## Registers Used in CPU SIM:

- Program Counter (PC) – Stores the address of the next instruction (12-bit).

- Address Register (AR) – Stores the memory address being accessed (12-bit).

- Instruction Register (IR) – Holds the fetched instruction (16-bit).

- Accumulator Register (AC) – Used for arithmetic operations (16-bit).

- Data Register (DR) – Temporarily holds data being processed (16-bit).

- Temporary Register (TR) – Optional register for intermediate calculations (16-bit).

- Condition Registers:

  - E (Carry Bit) – Used for carry operations.

  - S (Status Register) – Used for halt operations.

  - I (Indexing Bit) – Specifies direct or indirect addressing.

## Setting Up Memory

- Open the Hardware Module > Register > RAM.

- Create a new memory module:

  - Size: 4096 words

  - Cell Size: 16 bits

  - Type: RAM

  - Format: Hexadecimal

- Save the configuration with .cpu extension.

Machine based on the following architecture

Figure 1 machine based on the following architecture

- Set the indexing from the left side.

Execute and then click on option and select indexing set it to left.

# Fetch Cycle:

The Fetch Cycle involves the following steps:

| 1 | AR <- PC |
| --- | --- |
| 2 | IR <- Main [AR] |
| 3 | PC - INCR |
| 4 | AR <- IR (4-15) |
| 5 | DECODE – IR |

- Transfer the address from PC to AR.

- Read the instruction from memory into IR.

- Increment PC to point to the next instruction.

- Extract the address part of the instruction and transfer it to AR.

- Decode the instruction for execution.

# Fetch Execution

Lab Tasks

- Write a detailed explanation of how the Fetch-Decode-Execute cycle works.

- Use a simple instruction as an example and describe each step.

- Explain the role of PC, AR, IR, AC and DR in your own words.

- What is the function of the Arithmetic Logic Unit (ALU) in CPU operations?

How does ALU interact with registers and memory?

- Create a new base machine and change the bit width of a register (e.g., make AC 8-bit instead of 16-bit)