Lab 8: Simulate Subtract Operation

# Learning Objectives:

- Understand Two’s Complement for subtraction.

- Perform subtraction using bitwise operations.

- Analyze step-by-step execution of subtraction in assembly.

- Apply low-level programming for arithmetic operations.

# Essential Tools in Our Lab:

- Computer System: The main machine

- CPU Sim: CPU Sim is a tool used for simulating simple CPU architectures, helping students understand processor design, instruction execution, and debugging.

Program

| START: |  |
| --- | --- |
| INP | ; Take first input from the user and store it in the accumulator |
| STA NUM | ; Store the value from the accumulator into memory location NUM |
| INP | ; Take second input from the user and store it in the accumulator |
| CMA | ; Complement (invert) all bits of the second input (Two's complement preparation for subtraction) |
| INC | ; Add 1 to the complemented value (Completing Two's complement to get negative of the second input) |
| ADD NUM | ; Add the stored first input (NUM) with the negated second input (Effectively performing subtraction: First Input - Second Input) |
| OUT | ; Output the result of the subtraction operation |
| HLT | ; Halt execution |
| NUM: .data 1 0 | ; Memory location to store the first input, initially set to 0 |

# Basic Machine Instructions

Define Microinstructions for CMA and INC

Since we previously created microinstructions for addition, we will follow a similar approach but add CMA and INC instructions.

- Navigate to the Microinstructions Section in your simulator/tool.

# Define CMA (Complement Accumulator) Microinstruction:

  - This instruction will perform bitwise complement (logical NOT) on the Accumulator (AC).

  - Implementation: AC ← AC'

# Define INC (Increment Accumulator) Microinstruction:

  - This instruction increments the value in the Accumulator by 1.

  - Implementation: AC ← AC + 1

# Add CMA Instruction to the Machine Instruction Set

- Set the Opcode for CMA as E200.

- Set it as a Register Instruction.

- Go to Implementation and select Logical Instruction with the operation AC ← AC'.

- Define a Sequence Instruction for execution.

# Add INC Instruction to the Machine Instruction Set

- Set the Opcode for INC as E020.

- Set it as a Register Instruction.

- Select Increment Instruction with operation AC ← AC + 1.

- Define a Sequence Instruction for execution.

LAB TASKS

Task 1:

Accept three numbers as input and subtract them.

### Task 2:

### Subtract Two Numbers Stored in Memory (Predefined Values).

Task 3:

Store the result of subtraction in a memory location.